Practical 1, Part A: Automatic Alarm System

Code:

int sensorPin = 2; // PIR sensor connected to digital pin 2

int buzzerPin = 3; // Buzzer connected to digital pin 3

void setup() {

pinMode(sensorPin, INPUT);  // Set sensor pin as input

pinMode(buzzerPin, OUTPUT); // Set buzzer pin as output

}

void loop() {

if (digitalRead(sensorPin) == HIGH) { // Motion detected

digitalWrite(buzzerPin, HIGH);      // Turn on the buzzer

} else {

digitalWrite(buzzerPin, LOW);       // Turn off the buzzer

}

}

Practical 1, Part B: Timer-Based Buzzer in Tinkercad

int buzzerPin = 3;  // Buzzer connected to digital pin 3

void setup() {

pinMode(buzzerPin, OUTPUT);  // Set buzzer pin as an output

}

void loop() {

digitalWrite(buzzerPin, HIGH);  // Turn on the buzzer

delay(5000);                    // Keep it on for 5 seconds (5000ms)

digitalWrite(buzzerPin, LOW);   // Turn off the buzzer

delay(10000);                   // Wait for 10 seconds before triggering again

}

Practical 1, Part C: Sensor-Based Counting Device in Tinkercad

int sensorPin = 2;         // Sensor signal pin connected to digital pin 2

int count = 0;             // Variable to store the count

int previousState = LOW;   // Store the previous state to detect transitions

void setup() {

pinMode(sensorPin, INPUT);     // Set sensor pin as input

Serial.begin(9600);            // Initialize serial communication

}

void loop() {

int sensorState = digitalRead(sensorPin);  // Read the sensor state

// Detect a rising edge (LOW to HIGH transition)

if (sensorState == HIGH && previousState == LOW) {

count++;  // Increment the count

Serial.print("Count: ");

Serial.println(count);  // Display the updated count on the serial monitor

}

previousState = sensorState;  // Update the previous state

}

Practical 2: Demonstrate Communication Between Two Embedded Devices Using UART Port

Arduino 1 Code (Transmitter):

void setup() {

Serial.begin(9600);  // Start serial communication at 9600 baud rate

}

void loop() {

Serial.println("Hello from Arduino 1");  // Send a message

delay(2000);  // Wait for 1 second before sending the next message

}

Arduino 2 Code (Receiver):

void setup() {

Serial.begin(9600);  // Start serial communication

}

void loop() {

if (Serial.available() > 0) {  // Check if data is available

String receivedData = Serial.readString();  // Read the incoming data

Serial.print("Received: ");

Serial.println(receivedData);  // Print the received message to the serial monitor

}

}

Practical:-3 Ticket System

Code:-

#include <LiquidCrystal.h>

// Initialize the LCD (RS, E, D4, D5, D6, D7)

LiquidCrystal lcd(7, 8, 9, 10, 11, 12);

// Button pin

const int buttonPin = 2;

int buttonState = 0;

void setup() {

lcd.begin(16, 2);  // Initialize the LCD

pinMode(buttonPin, INPUT);  // Set button pin as input

lcd.print("Ticket System");

delay(2000);  // Display initial message for 2 seconds

lcd.clear();

lcd.print("Press Button");

}

void loop() {

buttonState = digitalRead(buttonPin);  // Read button state

if (buttonState == HIGH) {  // If button is pressed

lcd.clear();

lcd.print("Ticket Sent!");

delay(3000);  // Hold message for 3 seconds

lcd.clear();

lcd.print("Press Button");

}

}

Practical:-4 Movement & Orientation of Phone

#include <Servo.h>

#include <SoftwareSerial.h>

int pos = 0;

int sensorValue = 0;

int i = 0;

Servo servo_9;

void setup(){

servo_9.attach(9);

pinMode(A0, INPUT);

pinMode(13, OUTPUT);

Serial.begin(9600);

}

void loop(){

sensorValue = analogRead(A0);

pos = map(sensorValue,0,1023,0,180);

for (i = 0; i <= pos; i++) {

if(i == 90){

Serial.println("Landscape");

} else if( i == 180 || i == 0){

Serial.println("Portrait");

}

servo_9.write(i);

delay(15);

}

for (i= pos; i >= 0; i --) {

if(i == 90){

Serial.println("Landscape");

}

servo_9.write(i);

delay(15);

}

}

Practical:-5

Practical:-6 Water level in tank

// HC-SR04 Pins

const int trigPin = 9;

const int echoPin = 10;

// LEDs

const int redLED = 2;

const int yellowLED = 3;

const int greenLED = 4;

long duration;

int distance;

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

Serial.begin(9600);

}

void loop() {

// Send ultrasonic pulse

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

// Read echo

duration = pulseIn(echoPin, HIGH);

// Distance in cm

distance = duration * 0.034 / 2;

Serial.print("Distance = ");

Serial.print(distance);

Serial.println(" cm");

// Water Level

if (distance > 20) {        // Tank almost empty

digitalWrite(redLED, HIGH);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

}

else if (distance > 10) {   // Medium level

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, HIGH);

digitalWrite(greenLED, LOW);

}

else {                      // Tank full

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, HIGH);

}

delay(500);

}

Practical:-7 Intensity of light

Code:-

int sensorvalue = 0;

void setup()

{

pinMode(A0, INPUT);

Serial.begin(9600);

pinMode(9,OUTPUT);

}

void loop()

{

sensorvalue=analogRead(A0);

Serial.println(sensorvalue);

analogWrite(9,map(sensorvalue,0,1023,0,255));

delay(100);

}

Practical:-8 Motion detection

Code:-

int buzz=3;

int pir=2;

void setup()

{

pinMode(buzz, OUTPUT);

pinMode(pir, INPUT);

Serial.begin(9600);

}

void loop()

{

int status=digitalRead(pir);

if(status==HIGH)

{

Serial.println("Motion detectev");

tone(buzz,1000,700);

delay(2000);

}

else

{

Serial.println("No one is there");

delay(1000);

}

}